Appendix

## Beamsplitter Polarity

### Trade between reflecting and transmitting the science (IR) light

In the following MathCAD documents we evaluate the trade between having the beamsplitter transmit the science IR light (and hence reflect the WFS optical light), or alternatively reflect the science IR light (and hence transmit the WFS optical light). It was initially thought that it would almost certainly be better to reflect the science IR light, and thereby reduce the amount of emissivity of the science path. However, it was decided to check this assumption when it was realised that the best beamsplitters that transmit optical and reflect IR transmit only about 50% of the optical flux hitting them. Beamsplitters of the opposite ‘sex’ can transmit >96% of the IR light, while reflecting similar amounts of the optical. The trade then becomes one between whether it is better to have higher WFS signal, (and hence better image quality) at the cost of higher emissivity, or alternatively to trade WFS signal and image quality to get lower emissivity.

In the end it was decided to parameterise this trade in terms of the exposure time to obtain identical S/N on a point source in either case. If transmitting the IR science light, one then gets a better image quality, and more signal within a diffraction limited core, but a higher background. If reflecting the IR science light, there is poorer image quality, but a lower background.

In the MathCAD document, we show that for the expected beamsplitter performances, and using B. Ellerbroek’s curves for delivered Strehl versus guide star magnitude, one derives ratios of image quality (Strehl) which are a function of the initial guide star magnitude – ranging from equal for bright stars, to better by a factor 2 for the worst case.

We tabulate then the ratio of exposure times needed for equal point source S/N for two cases: (1) Where a star was chosen that gave ½ of the peak Strehl in the H-band. This choice means that the guide star produces rather poor Strehl in J, and rather high Strehl in K through M bands. The exposure times are then shorter for J-K bands for the IR transmit case, and essentially equal for the L and M band case. (2) Here a different star was chosen for each band – one that gave ½ of the peak Strehl in that band. For this case, the exposure time advantage is with the IR transmit case for all bands, although it ranges from 2.3 times shorter for J through to 1.3 times shorter for M.

As a result of these calculations, we have adopted an IR transmitting beamsplitter design. We note that the Keck AO team have made a similar decision, and that their decision partly drove the decision to perform this study.

It was also realised that GAOS performance, and especially it’s emissivity contribution would depend critically on the beamsplitter meeting the model performance. As a result, a sample was ordered, and it’s delivered performance is given earlier in this PDR documentation.

### Beamsplitter Polarity, Resulting Wavelength Dependence of WFS Throughput

The following 3 pages contain MathCAD calculation of the wavelength dependence of the photons detected by the AOWFS CCD. The assumptions include that the light comes from a K0 type star, that there are 6 silver reflection prior to the beamsplitter, the modeled throughput of the Lumonics beamsplitter and the CCD sensitivity curve. The wavelength dependence of the WFS optics after the beamsplitter are not included in the current curve. It is assumed that they will be essentially flat over the wavelength of interest.

As can be seen from the final plot, even for a star as red as K0, and with 6 silver reflection, a significant fraction of the photons detected by the WFS come from wavelengths shortward of 5500A. This information is used in later MathCAD calculations during the studies on the need for a WFS ADC.