ExAOC optical requirements

Comments by Bruce Macintosh, August 2, 2004

Responses by Brian Bauman, August 2, 2004

| Category | Description | Requirement | Comments |
| --- | --- | --- | --- |
| General requirements | Telescope interface | As given in Gemini ICD’s |  |
|  | Environmental | -10C to 10C; <50% relative humidity at MEMS | Is it the relative humidity that is important, or the dew point? need to verify environmental requirements at CP |
|  | Gravity vector | Up to 60 zenith angle, any azimuth angle |  |
| General optical requirements | Wavelengths | 0.7-0.9 WFS band, 1.0-2.4 aggregate science band (about 20% bandwidth at any one time); allow refocus after changing filter Goal: AO system allows science out to 4.2 microns |  |
|  | Field size | Goal: 5x5 arcsec square; requirement: 4x4 arcsec square |  |
|  | Pupil positions | Telecentric (pupil at infinity) for all non-collimated spaces | Ben would like to have a real 15mm pupil before the f/64 focus at the coronagraph input. Probably do-able.. |
|  | Throughput, uniformity for warm surfaces | >50% throughput, 1.0-2.4; >50% throughput, 0.7-0.9; <5% non-uniformity of throughput across pupil, 1.0-2.4 | 8 surfaces @ 92% reflectivity => 50% throughput; new protected silver has 96% reflectivity; Optosigma claims better. May be practical to check into LLNL durable silver coating. Coating two 4” mirrors is in the neighborhood of $7000. |
| AO relay requirements | Focal ratios (f-numbers) | f/16 at input of AO relay, f/64 at input of coronagraph, f/128 at science CCD | I believe Gemini is f/16. Whoops. I knew that. Coronagraph appears to want slower beam. I think that f/64 was asked for? |
|  | Tip-tilt correction location | Goal: tip/tilt correction performed at pupil plane Requirement: meet the centering requirement below |  |
|  | Tip-tilt range | 2.5 arcsec on sky | For 20mm pupil,  1 arcsec on sky translates to 1 mrad surface tip/tilt, which matches PI stage specs Could be descoped to 2-3 arcseconds The “plus/minus 1” spec was rendered as “61” in Bruce’s version, which is why he suggested a descope! Besides atmospheric tip/tilt, we need to be sure that we can handle whatever the Gemini telescope shake is PI has a new model that goes to 2.5 mrad mechanical: S-325. Data sheet at www.physikinstrumente.de /pdf/S325_Datasheet.pdf 2kHz resonant frequency unloaded, 1 kHz with 25mm x 8mm glass mirror on it. Cf. S-330 stage (1 mrad mechanical, 3.3kHz unloaded, 2.2kHz with same mirror), S-226 (1.1 mrad mechanical, 9 kHz unloaded, 7.5 kHz with 15mm diameter, 4 mm thick glass mirror. Contact at PI : (Roland Schreier, 714-850-9305); |
|  | Centering range | TBD% of telescope pupil | For alignment of main telescope into instrument—need to look at telescope documents or ask Gemini |
|  | MEMS size | 19.2 mm square | =64 * 300 |
|  | Beam size on MEMS | 61 actuator pitches across = 18.3mm | =61 * 300 |
|  | Common path wavefront error | < 25 nm integrated for aberrations with spatial frequency between 4/D and 32/D; 100nm rms integrated over spatial frequencies outside this range Stability: same | Simulations of intensity effects indicate that this is going to get tougher – 5-10 nm mid-frequency Is this correct for common path? |
|  | Non-common path wavefront error | < 5 nm integrated for aberrations with spatial frequency between 4/D and 32/D; 20nm rms integrated over spatial frequencies outside this range Stability: same |  |
| WFS requirements | Wavelength band | 0.7-0.9 | Some 0.8-0.9 light may be stolen by the science leg? |
|  | Number of subapertures | 62 subapertures wide across pupil diameter, outer 2 subapertures half-illuminated (i.e., beam footprint is 61 lenslet pitches) | 13.1 cm subapertures |
|  | WFS CCD | 128 x 128 pixel CCD with 24 pixels | 2x2 pixels per subaperture, no guard bands. Gemini has development effort for 256x256 array |
|  | Plate scale | 1.5 arcsec / pixel | Deserves to be revisited |
|  | Field | On-axis only; guide star will be steered onto WFS | Tolerance? |
|  | Field steering mechanism | Fold mirrors or fold mirror + translations |  |
| Coronagraph requirements | Alignment into coronagraph | Accuracy: 0.1 /D pointing, 0.1 subaperture centering Stability: 0.1 /D pointing, 0.1 subaperture centering over 10 minute exposure | Centering spec may be overkill, as suggested by Anand’s e-mail of 7/19 |
|  | Alignment/field steering mechanism | Fold mirrors or fold mirror + translations |  |
|  | Coronagraph/science camera configuration | Coronagraph and science camera are integrated together |  |
|  | Coronagraph pupil size and shape | 20 mm round | rather arbitrarily chosen |
|  | Wavefront error after occulting plane | 5 nm mid-frequency errors | Requirement in error budget is 5 nm mid-frequency errors |
| Science CCD requirements | Science CCD FOV | 4x4 arcsec? | 58 mm square chip?! Science focal plane size is lenslet array size. Still sounds big. |
| Observing requirements | Nodding | Nod science leg relative to WFS leg by up to 3 arcsec | Watch out for changes in non-common path error. Alternative is to operate AO loop off-null There is an additional requirement for steering of WFS with respect to science leg due to differential refraction.. In the absence of an ADC this would be ~2 arcseconds, could all come from steering the WFS. |
|  | Wavelength filters | TBD wavelength filters + 1 open + 1 closed; locate near pupil |  |
|  | Lyot stop | Translation: range/resolution |  |
|  | Observation procedure | TBD |  |
| Calibration requirements | Calibration hardware | SM fiber to be used in M-Z configuration—details TBD—other hardware required? |  |
|  | Calibration procedure | TBD |  |
| Metrology |  |  |  |